|  | # DHOLE PATIL EDUCATION SOCIETY’s # DHOLE PATIL COLLEGE OF ENGINEERING # Accredited by NAAC with A+ Grade, An ISO 9001:2015 Certified Institute, # 1284, Near Eon IT Park Kharadi, Dhole Patil College Road, Wagholi, Pune-412207 # Website: www.dpecoepune.edu.in E-mail: dpcoepune@gmail.com, Phone:020-66059900 |
| --- | --- |

DEPARTMENT OF FIRST YEAR ENGINEERING

ENGINEERING GRAPHICS

Projection of planes

ASSIGNMENT NO 2

- A thin square plate of side 40 mm stands on one of its corners in HP and the opposite corner is raised so that one of the diagonals is twice of the other. If one of the diagonals is parallel to both the reference planes, draw its projections and determine the inclinations of the plane with VP & HP.

- A rhombus of diagonals 60 mm and 40 mm having the longer diagonal parallel to the reference line represents the top view of a square lamina of diagonals 60 mm resting on a corner on the HP. Draw the projections and find inclinations of plane with VP and HP.

- An isosceles triangular plate of base side 50 mm & altitude 70 mm is resting in HP on its apex corner. Draw the projections if the side opposite to resting corner is 50 mm above HP & the plan altitude of triangle is inclined 300 with VP. Find inclination made by the plane with VP.

- A Triangular plane figure of sides 25 mm is resting on HP with one of its corners, such that the surface of the lamina makes an angle of 60 deg. with HP. If the side opposite to the corner on which the lamina rests makes an angle of 30 deg. with VP. Draw the top and front views in this position. Find inclination made by the plane with VP.

- Pentagonal lamina of edges 25 mm is resting on HP with one of its corners such that the plane surface makes an angle of 60 deg. with HP. The two of the edges containing the corner on which the lamina rests make equal inclinations with HP. When the edge opposite to the corner makes an angle of 45 deg. with VP and nearer to the observer. Draw the top and front views of the plane lamina in this position.

- A rectangular plane of edges 35 mm and 70 mm is resting on an edge in the HP. The surface is inclined to HP such that top view appears as a square. Draw its projections when the edge resting on HP is inclined at 30 degree to VP

- A hexagonal lamina of sides 25 mm rests on one of its corners on HP. The lamina makes 45 deg. to HP and the diagonal passing through the corner on which it rests is inclined at 30 deg. to VP. Draw its projections.

- A circular lamina of 50 mm diameter rests on HP such that one of its diameter is inclined at 30 deg. to VP and 45 deg. to HP. Draw its top and front views in this position

………………………………………….ALL THE BEST…………………………………………