# DHOLEPATILEDUCATIONSOCIETY'S

# DHOLEPATILCOLLEGEOFENGINEERING

AccreditedbyNAACwithA+Grade,AnISO9001:2015CertifiedInstitute

1284,NearEonITParkKharadi,DholePatilCollegeRoad,Wagholi,Pune-412207 Website:www.dpcoepune.edu.inE-mail:dpcoepune@gmail.com,Phone:020-66059900

| ACA/R / 44 | DepartmentofFirst Year Engineering | AcademicYear:2025-26 |
| --- | --- | --- |
| Rev: 00 |  | Semester: EVEN |
| Date: 15.12.2016 | CCE-IEXAM | Section: A AND B DIV |
|  |  | Subject: Basic Electronics Engg. |
| Duration:1Hr |  | Max.Marks: 30 |

Instruction:	1)Assume suitable data,if necessary.

2)Solve Q1 or Q2 and Q3 or Q4

| Question Number |  | Question | Marks | BT | CO |
| --- | --- | --- | --- | --- | --- |
| Q.1 |  | QUESTIONDISCRIPTION |  |  |  |
|  | a | Compare LED and Photodiode. | 04 | 1 | CO1 |
|  | b | What is extrinsic semiconductor. Explain P-type & N-type semiconductor. | 05 | 1 | CO1 |
|  | c | Draw and Explain Half Wave Rectifier (HWR) with its corresponding input and output waveforms | 06 | 3 | CO1 |
| Q.2 |  | QUESTIONDISCRIPTION |  |  |  |
|  | a | Define active and passive components. Explain them with suitable examples. | 04 | 2 | CO1 |
|  | b | Compare Half Wave rectifier (HWR), Full Wave rectifier (FWR) and Bridge rectifiers. | 05 | 3 | CO1 |
|  | c | Explain the operation of full wave Rectifier with suitable diagram and waveforms. | 06 | 3 | CO1 |
| Q.3 |  | QUESTIONDISCRIPTION |  |  |  |
|  | a | Explain impact of electronics on Industry. | 04 | 2 | CO1 |
|  | b | For a bridge rectifier, the RMS secondary voltage of transformer is 12.7 V. Assume ideal diodes and RL=1 k Ohm. Find : 1. Peak current 2. DC load current 3. DC load voltage 4. RMS current 5. Peak inverse voltage of diode 6. RMS voltage across load | 05 | 5 | CO1 |
|  | c | Explain the operation of Bridge Rectifier with suitable diagram and wave forms. | 06 | 3 | CO1 |
| Q.4 |  | QUESTIONDISCRIPTION |  |  |  |
|  | a | Explain the construction and working principle of LED | 04 | 2 | CO1 |
|  | b | A step down transformer having turns ratio 10:1 and input 230 V, 50 Hz is used in a half-wave rectifier. The diode forward resistance is 15 ohms and resistance of secondary winding is 10 ohms. For a load resistance of 4 k-ohms, calculate average and rms values of load current and voltage, rectification efficiency and ripple factor. | 05 | 5 | CO1 |
|  | c | Explain the construction and working of P - N junction diode. Draw its V - I characteristics. | 06 | 2 | CO1 |
|  |  | *******All the Best!******* |  |  |  |