|  | # DHOLE PATIL EDUCATION SOCIETY’s # DHOLE PATIL COLLEGE OF ENGINEERING # Accredited by NAAC with A+ Grade, An ISO 9001:2015 Certified Institute, # 1284, Near Eon IT Park Kharadi, Dhole Patil College Road, Wagholi, Pune-412207 # Website: www.dpecoepune.edu.in E-mail: dpcoepune@gmail.com, Phone:020-66059900 |
| --- | --- |

DEPARTMENT OF FIRST YEAR ENGINEERING

ENGINEERING GRAPHICS

Projections of Lines

- The point M of line MN is 20 mm above HP & is in VP. Its FV & TV makes 40° & 45° with HP & VP respectively. Draw the projections if the distance between the end projectors is 100 mm. Find true inclinations.

- The point A of line AB is 15 mm above HP and is in VP. The front view and top view of line AB makes 40° and 35° with XY respectively. Draw the projections; if the projector distance between the end points of the line is 60 mm. find the true length of line and true inclinations made by the line with HP & VP.

- The point B of line AB is in VP & its other end A is 20 mm above HP. Its FV makes 45° with HP & plan measures 60 mm. Draw the projections if the line is inclined to HP at 40°. Find TL & inclination made by the line with VP.

- The FV & TV of a line AB are contained by an auxiliary plane which is perpendicular to both references planes, measures 50 mm & 40 mm respectively. Draw the projections of a line if its end A is in HP while other end B is in VP.

- The point A of line AB is 15 mm above HP and is in VP. The front view and top view of line AB makes 40° and 35° with XY respectively. Draw the projections, if the end point B is 42 mm in front of VP. Find the true length of a line and true inclinations made by the line.

- A line AB is of length 75 mm has its end B 15mm above the HP and 50mm in front of the VP .End A is on the VP sum of inclination of the line with principal planes HP and VP is 90°. Draw the projection of the line .Find inclination of line with HP and VP.

- The distance between the end projectors of line AB is 60mm.The end A is 25mm above HP and 45mm in from VP .while the other end B is 60 mm above HP and 15 mm in front VP .Draw the projection and find the true length and also inclination of the line with HP and VP.

- The point M of line MN is in HP and 15mm in front of VP while its other end N is 50mm above HP. The line is inclined to VP at an angle of 40°.  Draw the projections of the line if its elevation makes 29° with HP. Find the True length & true inclination made by the line with HP.

…………………………………ALL THE BEST………………………………..